Predictive Maintenance & Fault Diagnosis

Four-Model Comparison Report

Consolidated technical summary of dataset structure, features, targets, model candidates, and final selected architectures across four independent predictive-maintenance systems.

| Model | Domain / Task | Rows / Timesteps | Samples / Units |
| --- | --- | --- | --- |
| Model 1 | UAV Telemetry Sensor Fault Detection & Diagnosis | ~8.8 Million rows | 70 Flights (~400K Windows) |
| Model 2 | NASA C-MAPSS Turbofan Engine RUL Prediction | 265,256 rows | 1,416 Engine Units |
| Model 3 | CWRU Bearing Fault Diagnosis | 5,750 patches | 5,750 Samples |
| Model 4 (A) | AI4I 2020 CNC Milling Machine Predictive Maintenance | 10,000 rows | 10,000 Operations |

Model 1 — UAV Telemetry Fault Detection

Task: Multi-Class Fault Classification (0 = Nominal, 1–6 = Fault Modes)

Total Rows: ~8,800,000 raw 20Hz telemetry timesteps across 70 flight sessions   |   Total Samples: ~400,000 sliding windows (4s @ 20Hz = 80 timesteps × 32 channels)

| I/O | Column Name | Sensor Domain | Expected Range (Min–Max) | Description |
| --- | --- | --- | --- | --- |
| Input | IMU_GyrX, IMU_GyrY, IMU_GyrZ | Gyroscope | -10.47 to +13.31 rad/s | Angular velocity along X, Y, Z axes |
| Input | IMU_AccX, IMU_AccY, IMU_AccZ | Accelerometer | -26.36 to +82.93 m/s² | Linear acceleration along X, Y, Z axes |
| Input | ATT_Roll, ATT_Pitch, ATT_Yaw | Attitude Estimate | Roll [-59.77°,50.01°]; Pitch [-38.60°,46.80°]; Yaw [0°,360°] | Vehicle orientation Euler angles |
| Input | ATT_ErrRP, ATT_ErrYaw | Attitude Error | 0.00 to 0.99 | EKF attitude estimation error variance |
| Input | XKF1_Roll, XKF1_Pitch, XKF1_Yaw | Extended Kalman Filter | Roll [-93.76°,59.95°]; Pitch [-49.88°,74.95°]; Yaw [0°,360°] | Filtered state trajectory estimates |
| Input | BARO_Alt, BARO_Press, BARO_Temp, BARO_CRt | Barometer | Alt [-5.38,115.60] m; Press [93.55,94.85] kPa; Temp [29.54,31.41] °C; Climb [-7.71,7.96] m/s | Altitude, pressure, temp & climb rate |
| Input | GPS_NSats, GPS_HDop, GPS_Spd, GPS_Alt | GPS Module | Sats [3,10]; HDop 1.21; Speed [0,18.91] m/s; Alt [553.14,668.18] m | Satellite count, precision, ground speed, alt |
| Input | MAG_MagX, MAG_MagY, MAG_MagZ | Magnetometer / Compass | -499 to +498 mG | Magnetic field strength X, Y, Z |
| Input | BAT_Volt, BAT_Curr | Battery Telemetry | Volt 12.58 V; Current [0.00,71.54] A | System supply voltage & motor current draw |
| Input | VIBE_VibeX, VIBE_VibeY, VIBE_VibeZ | Vibration Sensors | 0.00 to 21.94 m/s² | Airframe structural vibration levels |
| Input | MAV_rxp, MAV_txp | RC Telemetry Link | rxp [152,5697]; txp [39,65512] | Packets received/transmitted |
| Output | Status / Class Target | Classification Target | Values: 0,1,2,3,4,5,6 | 0: No Failure, 1: RC Fail, 2: GPS Fail, 3: Accel Fail, 4: Gyro Fail, 5: Compass Fail, 6: Baro Fail |

Model 2 — Turbofan Engine Remaining Useful Life (RUL) Prediction

Task: Regression (Predict remaining operational cycles before engine failure)

Total Combined Rows: 265,256 (Train: 160,359; Test: 104,897 across FD001–FD004)   |   Total Samples: 1,416 Turbofan Engine Units (709 Training, 707 Testing)

| I/O | Column Name | Feature Type | Value Range | Units / Description |
| --- | --- | --- | --- | --- |
| Input | setting_1 | Altitude | -0.009 to 41.99 | Flight altitude (kft) |
| Input | setting_2 | Mach Number | -0.001 to 0.840 | Mach speed |
| Input | setting_3 | Throttle Resolver Angle | 60 to 100 | TRA (%) |
| Input | s_1 | Fan Inlet Temperature | 445.00 to 549.00 | °R |
| Input | s_2 | LPC Outlet Temperature | 537.00 to 645.00 | °R |
| Input | s_3 | HPC Outlet Temperature | 1353.00 to 1616.91 | °R |
| Input | s_4 | LPT Outlet Temperature | 1048.00 to 1441.49 | °R |
| Input | s_5 | Fan Inlet Pressure | 2.10 to 14.62 | psia |
| Input | s_6 | Bypass-Duct Pressure | 8.40 to 21.61 | psia |
| Input | s_7 | HPC Outlet Pressure | 138.00 to 556.06 | psia |
| Input | s_8 | Physical Fan Speed | 1973.00 to 2388.56 | rpm |
| Input | s_9 | Physical Core Speed | 7892.00 to 9244.59 | rpm |
| Input | s_10 | Engine Pressure Ratio | 0.999 to 1.470 | EPR |
| Input | s_11 | HPC Outlet Static Pressure | 46.85 to 97.30 | psia |
| Input | s_12 | Fuel Flow Ratio | 518.69 to 1167.00 | pps/psi |
| Input | s_13 | Corrected Fan Speed | 1895.00 to 2388.56 | rpm |
| Input | s_14 | Corrected Core Speed | 7382.00 to 8293.72 | rpm |
| Input | s_15 | Bypass Ratio | 5.660 to 8.585 | Ratio |
| Input | s_16 | Burner Fuel-Air Ratio | 0.028 to 0.038 | Ratio |
| Input | s_17 | Bleed Enthalpy | 388.00 to 400.00 | lb/s |
| Input | s_18 | Required Fan Speed | 1973 to 2388 | rpm |
| Input | s_19 | Required Fan Conversion Speed | 100.0 | % |
| Input | s_20 | HP Turbine Cool Air Flow | 6.50 to 39.43 | lb/s |
| Input | s_21 | LP Turbine Cool Air Flow | 14.00 to 23.62 | lb/s |
| Output | RUL | Target Regression Variable | 0 to 303 | Remaining Useful Life in operating cycles |

Model 3 — Bearing Fault Diagnosis (CWRU Vibration Dataset)

Task: 10-Class Vibration Signal Fault Classification & Severity Isolation

Total Rows / Samples: 5,750 samples (4,600 Train + 1,150 Test, balanced 80/20 split)

Inputs are raw continuous vibration telemetry from CWRU bearing accelerometer sensors under 1 HP motor load (~1772 RPM) at 48 kHz sampling frequency, sliced into 32×32 image-like patches for 2D CNN feature extraction. Mean value across the dataset is +0.0157.

| Data Component | Variable Name | Shape / Dimensions | Data Type | Value Range |
| --- | --- | --- | --- | --- |
| Training Input | X_train | (4,600, 32, 32) or (3,680, 32, 32, 1) | float64 | -6.2926 to +6.8259 |
| Training Output | y_train | (4,600,) or (3,680,) | int64 / str | 0 to 9 (or string tags) |
| Class ID (y) | String Label | Fault Location / Component | Damage Diameter | Sample Count |
| 0 | Normal | Healthy / Baseline | No Fault | 460 |
| 1 | IR_007 | Inner Race Fault | 0.007 in (0.18 mm) | 460 |
| 2 | IR_014 | Inner Race Fault | 0.014 in (0.36 mm) | 460 |
| 3 | IR_021 | Inner Race Fault | 0.021 in (0.53 mm) | 460 |
| 4 | Ball_007 | Rolling Element (Ball) Fault | 0.007 in (0.18 mm) | 460 |
| 5 | Ball_014 | Rolling Element (Ball) Fault | 0.014 in (0.36 mm) | 460 |
| 6 | Ball_021 | Rolling Element (Ball) Fault | 0.021 in (0.53 mm) | 460 |
| 7 | OR_007 | Outer Race Fault | 0.007 in (0.18 mm) | 460 |
| 8 | OR_014 | Outer Race Fault | 0.014 in (0.36 mm) | 460 |
| 9 | OR_021 | Outer Race Fault | 0.021 in (0.53 mm) | 460 |
| Total | — | 10 Balanced Classes | — | 4,600 |

Model 4 — Auxiliary Predictive Maintenance (Milling Machine & Engine Health)

Sub-Dataset A: AI4I 2020 CNC Milling Machine Dataset

Total Rows / Samples: 10,000 machining operations

| I/O | Column Name | Type | Value Range | Description |
| --- | --- | --- | --- | --- |
| Input | Type | Categorical | L, M, H | Quality variant grade (Low, Medium, High) |
| Input | Air temperature [K] | Continuous | 295.3 to 304.5 K | Ambient air temperature |
| Input | Process temperature [K] | Continuous | 305.7 to 313.8 K | Machining process temperature |
| Input | Rotational speed [rpm] | Continuous | 1,168 to 2,886 rpm | Spindle rotational speed |
| Input | Torque [Nm] | Continuous | 3.8 to 76.6 Nm | Motor torque output |
| Input | Tool wear [min] | Continuous | 0 to 253 min | Accumulated tool operating time |
| Output | Machine failure | Binary Target | 0 or 1 | 0: Operational Nominal, 1: Failure Occurred (~3.4% failure rate) |
| Output | TWF, HDF, PWF, OSF, RNF | Multi-label Flags | 0 or 1 | Tool Wear, Heat Dissipation, Power, Overstrain & Random Failures |

Combined Summary — All 4 Models

Model Comparison — Updated Summary (Model 4 Revised). Models 1–3 unchanged from prior summary; Model 4 reflects the Auxiliary Predictive Maintenance (AI4I 2020) Final System Report — two-stage Balanced Random Forest architecture.

Table 1 — Candidate Models & Final Selection

| Model | Candidates Trained | Selection Rule | Model Chosen | Result Achieved |
| --- | --- | --- | --- | --- |
| Model 1 – UAV Fault Detection | Two-Stage LightGBM (ML) vs. Two-Stage CNN-BiLSTM (DL) | Classification → highest accuracy | LightGBM | 94.48% accuracy (vs. 70.98% for CNN-BiLSTM) — Macro F1 92.07% |
| Model 2 – RUL Prediction | XGBoost vs. CNN-LSTM vs. XGBoost+CNN-LSTM Ensemble | Regression → lowest MAE | XGBoost | MAE 9.55 cycles (vs. 10.21 Ensemble, 13.15 CNN-LSTM) — final tuned MAE 9.58 cycles |
| Model 3 – Bearing Health | CNN only (single architecture documented, no comparison run) | Classification → highest accuracy | CNN | 98.99% accuracy — Macro F1 98.99%, Test Loss 0.0672 |
| Model 4 – Auxiliary / AI4I | Balanced Random Forest (Stage 1); LightGBM + SMOTE / Rule-Based (Stage 2) | Stage 1: Classification → highest accuracy + best precision/recall balance. Stage 2: per-failure-type → highest accuracy/F1 (rule fallback where <50 positive cases) | Balanced Random Forest (Stage 1, class_weight=balanced) + LightGBM/SMOTE & rule-based models (Stage 2) | Stage 1: 98.90% accuracy, ROC-AUC 0.9691, Precision 85.94%, Recall 80.88%, F1 0.8333. Stage 2: HDF 100%, PWF 100%, OSF 99.80% (F1 0.8947), TWF 98.45% (rule), RNF 99.75% (rule) |

Table 2 — Input / Target / Dataset / Source by Model

| Attribute | Model 1 – UAV Fault Detection | Model 2 – RUL Prediction | Model 3 – Bearing Health | Model 4 – Failure Detection |
| --- | --- | --- | --- | --- |
| Input Features | 32 channels → 465 features | 24 cols → 216 features | 32×32 vibration image | 6 raw sensor readings |
| Target | Fault class ID 0–6 (No Failure, RC, GPS, Accelerometer, Gyro, Compass, Barometer failure) | RUL (cycles remaining) | Bearing fault class (Normal, Ball, Inner Race, Outer Race at 3 severities) | Binary failure status (Healthy / Failure) + 5 failure-type labels (TWF, HDF, PWF, OSF, RNF) |
| Dataset | ALFA UAV telemetry | NASA C-MAPSS (FD001–FD004 combined) | CWRU (Case Western Reserve University) bearing dataset | AI4I 2020 dataset (ai4i2020.csv, 14 cols; ai4i2020-selected-columns.csv, 10,000 records) |
| Source | ALFA UAV telemetry / MAVLink flight-controller logs (Pixhawk/ArduPilot/PX4) | NASA C-MAPSS turbofan degradation simulation dataset | CWRU bearing vibration dataset | AI4I 2020 predictive maintenance dataset (UCI Machine Learning Repository) |